| NORAC SIGNAL RULE AS APPLIED TO MSTS COLORLIGHT SIGNALS: |  |  |  |  |
| --- | --- | --- | --- | --- |
| ASPECTS | ## RULE | ## NAME | ## INDICATION | ## MSTS TOKEN |
|  | # 281 | ## CLEAR | ## Proceed not exceeding Normal Speed | ## CLEAR_2 |
|  | 281b | APPROACH LIMITED | Proceed approaching the next signal at Limited Speed | APPROACH_3 |
|  | 281c | LIMITED CLEAR | Proceed at Limited Speed until entire train clears all interlocking or spring switches, then proceed at Normal Speed. | CLEAR_1 |
|  | 282 | APPROACH MEDIUM | Proceed approaching next signal at Medium Speed. | APPROACH_2 |
|  | 282a | ADVANCE APPROACH | Proceed prepared to stop at the second signal. Trains exceeding Limited Speed must begin reduction to Limited Speed as soon as engine passes the Advance Approach signal | APPROACH_2 |
|  | 283 | MEDIUM CLEAR | Proceed at Medium Speed until entire train clears all interlocking or spring switches, then proceed at Normal Speed. | CLEAR_1 |
|  | 284 | APPROACH SLOW | Proceed approaching next signal at Slow Speed. Trains exceeding Medium Speed must begin reduction to Medium Speed as soon as engine passes the Approach Slow signal. | APPROACH_2 |
|  | 285 | APPROACH | Proceed prepared to stop at the next signal. Trains exceeding Medium Speed must begin reduction to Medium Speed as soon as the engine passes the Approach signal. | APPROACH_1 |
|  | 286 | MEDIUM APPROACH | Proceed prepared to stop at the next signal. Trains exceeding Medium Speed must begin reduction to Medium Speed as soon as the Medium Approach Signal is clearly visible. | APPROACH_1 |
|  | 287 | SLOW CLEAR | Proceed at Slow Speed until entire train clears all interlocking or spring switches, then proceed at Normal Speed | CLEAR_1 |
|  | 288 | SLOW APPROACH | Proceed prepared to stop at the next signal. Slow Speed applies until entire train clears allinterlocking or spring switches, then Medium Speed applies | APPROACH_1 |
|  | 290 | RESTRICTING | Proceed at Restricted Speed | RESTRICTING |
|  | 291 | STOP AND PROCEED | Stop, then proceed at Restricted Speed | STOP_AND_ PROCEED |
|  | 292 | STOP | Stop | STOP |